SITE ENGINEER GRADE II,FILM DEVELOPMENT CORPORATION

EXAM DATE:06-10-2015

1.Any trap should have:

A.Water seal*

B.Sullage

C.A bend

D.Grating

Ans:A

2.Eco system is:

A.Hatbitat

B.Communit=y and environment*

C.Community

D.Biosphere and habitat

Ans:B

3.The outer signal is provided at a minimum distance of:

A.1 km from home signal

B.580 m from home signal*

C.860 m from home signal

D.180 m from home signal

Ans:B

4.A thin cylinder of diameter 100 m and thickness 5 mm is subjected to a internal fluid subjected to a internal fluid pressure of 10N/mm2.The hoop stress is:

A.150 N/mm2

B.10 N/mm2

C.15 N/mm2

D.100 N/mm2 *

Ans:D

5.In a theodolite traversing the angular error should not exceed using 20” least count theodolite and ‘n’ number of sides:

A.20”Square root of  n*

B.20”xN2

C20”xSquare root of n2

D.20”x2n

Ans:A

6.A horizontal force 40 kgf is applied on a body of weight 120 kg placed in a horizontal plane.If the body is just in the point of motion,the angle of friction be:

A.200

B.180 26*

C.100

D.250

Ans:B

7.An equation for a deficient frame:

A.n=2j+3

B.n=2j-3*

C.n=j+3

D.n=j-3

Ans:B

8.The process of applying cement mortar under pressure through a nozzle is called:

A.Pressurising

B.Prestressing

C.Guniting*

D.Compressing

Ans:C

9.An example of endogenous tree is:

A.Bamboo*

B.Teak

C.Deodar

D.Oak

Ans:A

10.The purpose of soundness list is to determine:

A.The presence of free lime*

B.Setting time

C.Sound proof quality of cement

D.The fineness

Ans:A

11.The maximum permissible excentricity of load on a rectangular foundation with width B is equal to:

A.B/3

B.B/6

C.B/2

D.B/4*

Ans:D

12.The portion of the brick obtained by cutting into two portion longitudinally is called:

A.Bat

B.King closer

C.Queen closer*

D.Beveled closer

Ans:C

13.The inner curve of an arch is:

A.Spandril

B.Extradose*

C.Soffit

D.Arcade

Ans:B

14.Life became essential in a building when the number of floors exceeds:

A.2

B.3

C.4*

D.6

Ans:C

15.The moment of inertia of a rectangular section having b-width and d-depth about x-axis is given by:

Ans.bd3/12

16.The ratio of volume of voids to the total volume of given soil is:
A.Voids ratio

B.Porosity*

C.Air volume

D.Air area

Ans:B

17.A pull of 20 t is suddenly applied to a road of cross-sectional area 40 cm2.The stress produced in the rod is equal to:

A.0.5 t/cm2

B.1.0 t/cm2

C.2.0 t/cm2 *

D.4 t/cm2

Ans:C

18.In a fixed beam,the slopes at the ends are:

A.Minimum

B.Maximum

C.Same as at centre

D.Zero*

Ans:D

19.Adesirable pH value for water is:

A.7

B.6 to 8

C.5 to 9

D.7 to 8.5*

Ans:D

20.The magnetic bearing of a line AB is 212030’ and the declination 2015’ east.What is true bearing of the line?

A.210015’

B.214045’*

C.210045’

D.132030’

Ans:B

21.The process of proper and accurate measurement of concrete ingredients for uniformity proportion is known as:

A.Batching*

B.Grading

C.Mixing

D.Blending

Ans:A

22.The distance to the neutral axis from the top compression fibre of a singly reinforced rectangular beam is:

Ans:Xu=(0.87 fy Ast)/0.36 fck b

23.The bearing stiffeners in plate girders are provided at:

A.Mid span

B.Quarter points

C.Supports*

D.Equal intervals

Ans:C

24.The most economical section

A.Square

B.Circular

C.Channel

D.Tubular*

Ans:D

25.Bearing of OA=23030’ and bearing of OB=120000’,DAOB is:

A.99030’*

B.100030’

C.140030’

D.280000’

Ans:A

26.The value of dismantled material less the cost of dismantling is called:

A.Scrap value*

B.Salvage value

C.Book value

D.Marhit value

Ans:A

27.Back washing is highly effective in case of:

A.Slow Sand filters

B.Rapid sand filters*

C.Pressure filters

D.Rapid and slow sand filters

Ans:B

28.Colour of fresh sewage:

A.Brown

B.Gray*

C.Pink

D.Black

Ans:B

29.If’W’is the weight per volume,’P’safe bearing capacity and  is the angle of repose of soil retained by the retaining wall.The minimum depth of foundation is:

Ans:

30.The bending moment in a beam will be maximum where:

A.The shear force is uniform

B.The shear force is maximum

C.The shear force is zero*

D.The shear force is minimum

Ans:C

31.A steel wire of 5 mm diameter is bend into a circular shape of 5 m radius.Young’s modulus E=200 GPa.The maximum stress induced in the wire is:

A.100 N/mm2 *

B.100 N/mm2

C.10 N/mm2

D.1 N/mm2

Ans:A

32.A unit is working in anaerobic action is:

A.Activated sludge process

B.Tricling filter

C.Contact bed

D.Septic tank*

Ans:D

33.The primary function of sleeper is:

A.To maintain gauge*

B.To take loads from rails

C.To give cushioning action

D.To give stability of the train

Ans:A

34.ABCD is a square.The bearing of AB is 500.the bearing  of DC is:

A.1400

B.1900

C.500 *

D.2900

Ans:C

35.The sum of Interior angles of a closed traverse is:

A.(2n-4)90*

B.(2n+4)90

C.(2n-4)180

D.(2n+4)180

Ans:A

36.Current meter is used for measuring:

A.Velocity*

B.Viscosity

C.Current

D.Pressure

Ans:A

37.The quantity of cement required for the ratio of 1:5 in cement mortar is:

A.288 kg*

B.240 kg

C.178 kg

D.206 kg

Ans:A

38.In a problem the algebraic sum of all horizontal (aH)is 16 N and the algebraic sum of all vertical force(aV)is 32 N.The resultant force is:

A.48.5 N

B.35.8 N*

C.38.5 N

D.40 N

Ans:B

39.The process of adding water to lime and convert it into hydrated lime is called:

A.Watering

B.Baking

C.Hydration

D.Slaking*

Ans:D

40.Francis turbine is:

A.Impulse turbine

B.Radial flow impulse turbine

C.Axial flow turbine

D.Reaction radial flow turbine*

Ans:D

41.A curve consists of two arcs of equal or different radii bending in opposite direction is:

A.Simple curve

B.Compound curve

C.Reverse curve*

D.Transition curve

Ans:C

42.The latitude and departure of any point with reference to the preceding point is known as:

A.Independent co-ordinate

B.Consecutive co-ordinate*

C.Total latitude

D.Total departure

Ans:B

43.Bemoullis theorem deals with the law of conservation of:

A.Mass

B.Momentum

C.Energy*

D.Velocity

Ans:C

44.For a simply supported beam,the maximum deflection permitted is:

A.1/300 of the span

B.1/325 of the span*

C.1/350 of the span

D.1/400 of the span

Ans:B

45.In a cantilever retaining wall for height’H’,the horizontal pressure of the earth will act at a distance of:

A.H/3 from the top

B.H/3 from the base*

C.H/2 from the top

D.H/4 from the base

Ans:B

46.Core-cutter method is used for:

A.Determining density of soil*

B.Obtaining samples for direct shear test

C.Determining bearing capacity of soil

D.Compacting soil

Ans:A

47.The indentation provided in a face of the brick is called:

A.Plank

B.Pallet

C.Striker

D.Frog*

Ans:D

48.Inlimit state method of design,the over reinforced sections:

A.Are not permitted*

B.Are permitted

C.Are permitted only in extreme fibres

D.Permitted in any case

Ans:A
49.Polar moment of inertia of a solid shaft of diameter D is:

Ans:(Pie/32)*D4

50.A chain 20 m long is 20 cm too short was used to measure a line and the result was 190 m.The true length is:

A.188.1 m*

B.191.5 m

C.193 m

D.190 m

Ans:A

51.Combined correction for refraction and curvature of 1 km is:

A.0.0112 m

B.0.0673 m*

C.0.0785 m

D.0.0073 km

Ans:B

52.A 15 cm theodolite means:

A.Length of the telescope is 15 cm

B.Height of standards is 15 cm

C.Diameter of lower plate is 15 cm*

D.Radius of upper plate is 15 cm

Ans:C

53.CPM stands for:

A.Computer Programming Mode

B.Critical Project Management

C.Critical Path Method*

D.Controlling,Planning and Maintenance

Ans:C

54.The approximate weight of 1m3 of mild steel is:

A.1000 kg

B.240 kg

C.14000 kg

D.7850 kg*

Ans:D

55.In a closed travers the algebraic sum of deflection angle is:

A.00

B.3600 *

C.(2n+4)*9

D.(n-2)180

Ans:B

56.For a level section area of a  trapezoidal cut when base width ‘B’ side slope ‘S’ and depth’d’:

A.Bd+Sd2 *

B.Bd2+Sd

C.Bd-Sd2

D.Bd2-Sd

Ans:A

57.In a riveted joint,when the number of rivets decreses from the innermost to the outermost row the joint is said to be:

A.Chain

B.Zig-Zag riveted

C.Diamond riveted*

D.Single riveted

Ans:C

58.The best type of ballast is :

A. granite

B.Sandstone

C.Limestone

D.Quartzite*

Ans:D

59.A small reflecting instrument is used to fixing up intermediate points on the line is called:

A.Line ranger*

B.Ranging road

C.Open cross staff

D.Offset rod

Ans:A

60.Hydraulic ram is a pump which works:

A.On the principle of water hammer

B.On the principle of centrifugal action

C.Principle of reciprocating action*

D.Direct action

Ans:C

61.When water table is within the root zone depth and is determined to the plant life,the land is said to be:

A.Basin flooding

B.Water logged*

C.Over nourished

D.Super saturated

Ans:B

62.The best pipe for water mains for long life is:
A.Steel

B.Cement concrete

C.Cast iron*

D.Asbestos cement

Ans:C

63.The best system of a railway highway crossing is:

A.Level crossing

B.Road over rail track*

C.Road under rail track

D.Both road over and under bridges

Ans:B

64.The process of taking out stones from natural rock is:

A.Dressing

B.Seasoning

C.Quarrying*

D.Pitching

Ans:C

65.If the higher values are outside ,series of closed contours on a plane indicate a:

A.Hill

B.Depression*

C.Valley

D.Ridge

Ans:B

66.The surface of a still lake is an example of:

A.Horizontal surface

B.Datum surface

C.Level surface*

D.Horizontal plane

Ans:C

67.Two forces P and Q are acting at an angle q,their resultant R is given by:

Ans:R=Square root of P2+Q2+2PQ cos(theta)

68.The road connecting district headquarters of a state is:

A.State Highway*

B.National Highway

C.District  major roads

D.Minor district roads

Ans:A

69.Pile foundations are normally used:

A.In soft clayey soils

B.In heavy load situations

C.When the bearing area required is not available*

D.In loose sandy soil

Ans:C

70.A simply supported beam of span’I’carries a point load ‘w’ at its centre and rigidity  of beam is ‘El’.The maximum deflection at the centre is:

Ans:wl3/48El

71.The Headquarters of FIFA is located at--------

A.New York

B.Munich

C.Zurich*

D.London

Ans:C

72.Conaru Hampi’s name is associated with-----

A.Chess*

B.Badminton

C.Snooker

D.Tennis

Ans:A

73.The Chief Minister of Maharahtra is-----

A.Ashok Chavan

B.Deshmukh

C.Devendra Fadnavis*

D.Ajith Pawar

Ans:C

74.The 8th G-20 Summit held at -----------

A.New Delhi

B.Moscow*

C.Geneva

D.Sri Lanka

Ans:B

75.’Conversations with my life’ was written by--------

A.Man Mohan Singh

B.R.K.Narayan

C.P.V.Narasimha Rao

D.Nelson Mandela*

Ans:D

76.Poyikayil Yohannan’s name is associated with-----------

A.PRDS*

B.SYDD

C.Yogakshema Sabha

D.SNDP

Ans:A

77.The East flowing rivers of Kerala are the tributaries of River----------

A.Periyar

B.Bharathapuzha

C.Pampa

D.Kaveri*

Ans:D

78.The leader of ‘Malayalee Memorial’was-------

A.Dr.Palpu

B.G.P.Pillai*

C.N.Raman Pillai

D.R.Ranga Rao

Ans:B

79.Who among the following founded the organization ‘Samathwa Samajam’?

A.Vaikunda Swamikal*

B.Mannathu Padmanabhan

C.Sree Narayana Guru

D.Kumaranasan

Ans:A

80.Yogakshema Prasthanam started in the year-----------

A.1906

B.1908*

C.1912

D.1914

Ans:B

81.Lucknow city is situated on the banks of the river---------

A.Gomati*

B.Yamuna

C.Ganga

D.Padma

Ans:A

82.Which district in Kerala has the largest forest area?

A.Wayanadu

B.Idukki*

C.Malappuram

D.Kasarkodu

Ans:B

83.The first concrete dam in India is------

A.Bhakranangal

B.Hirakud

C.Mattupetti*

D.Malampuzha

Ans:C

84.Which temple is known as ‘Dakshina Kashi’?

A.Guruvayoor

B.Sabarimala

C.Vaikom Siva Temple*

D.Panachikadu Temple

Ans:C

85.Who among the following is known as’The father of Indian Renaissance’?

A.Bankim Chandra Chatterjee

B.Ram Mohan Roy*

C.Debendranath Tagore

D.Gopalakrishna Gokhale

Ans:B

86.SNDP was formed in the year-----------

A.1901

B.1903*

C.1905

D.1908

Ans:B

87.Who among the following was the first President of All India Trade Union Congress?

A.Tilak

B.Gokhale

C.Lala Lajpath Rai*

D.S.N.Banerjee

Ans:C

88.Communist Party of India was formed in the year---------

A.1920

B.1923

C.1925*

D.1927

Ans:C

89.’Kevala Devu National Park’ is situated at-----------

A.Rajasthan*

B.Uttarakhand

C.Punjab

D.Himachal Pradesh

Ans:A

90.Guruvayoor Sathyagraha was in the year-----------

A.1923

B.1924

C.1925

D.1931*

Ans:D

91.The standard dimension of brick as per Indian standard is:

A.20x10x10 cm

B.22.5x11.5x8 cm

C.19x9x9 cm*

D.18x9.5x9.5 cm

Ans:C

92.The aggregate is fine if it is completely retained on:

A.0.15 mm sieve*

B.0.30 mm sieve

C.4.75 mm sieve

D.1.18 mm sieve

Ans:A

93.One arc is:

A.1 m2

B.100 m2 *

C.1000 m2

D.10 m2

Ans:B

94.The WCB of a line OA is 1320 its guadrantal bearing is:

A.S 480E*

B.S 480W

C.N 3120W

D.E 480S

Ans:A

95.Chercy’s formula is given as:

Ans:V=C Square root of mi

96.The algebraic sum of the moments taken about any point in the plane of forces is zero is known as:

A.Law of polygon of forces

B.Lamis theorem

C.Newton’s law of force

D.Laws of moments*

Ans:D

97.The resultant hydrostatic force acts through a points is known as:

A.Centre of gravity

B.Centre of buoyancy*

C.Centre of pressure

D.Centroid

Ans:B

98.The centre of gravity of quadrant of circles is at a distance of:

Ans:4r/3(Pie) from the axis

99.If the Poisson’s ratio of a material is 0.25,the ratio of modulus of rigidity to the Young’s modulus is:

A.2

B.4

C.2.5

D.0.4*

Ans:D

100.The point of contraflexture is the point where:

A.Shear force is zero

B.The bending moment is Zero

C.Beam is supported

D.The bending changes its sign*

Ans:D