Study guide for water and energy balance of watersheds.

Vocabulary:

Hydraulic gradient, hydraulic head, elevation head, equipotential lines, flow lines, divergent flow, convergent flow, area normalized flow, areal yield, specific discharge, control volume, conservation equation, water balance, precipitation, evapotranspiration, storage, local groundwater flow, regional groundwater flow,  daily mean water balance, annual mean water balance, persistence, autocorrelation, water year, runoff ratio, energy, power, energy flux (area normalized power), energy balance, radiation, shortwave radiation, black body, backscatter, longwave radiation, back radiation, atmospheric window, greenhouse gas, greenhouse effect, sensible heat, latent heat, dynamic equilibrium

Concepts:

Understand how elevation head can be used to estimate flow direction.  Understand key assumptions about groundwater flow that are necessary to translate elevation gradients to hydraulic gradients.

Understand how to delineate and double-check a watershed definition on a topographic or relief map.

Understand how flow nets based on topography indicate divergent or convergent flow, and understand how those types of flow nets are related to fundamental geomorphologic process domains.

Understand how area-normalized flow (a.k.a. areal yield or specific discharge) affects hydrologic interpretation of comparisons of watersheds.

Understand how to define a conservation equation for a control volume.

Understand how to define a full water balance equation for an arbitrary control volume of the earth surface.

Understand some basic mechanisms by which groundwater may enter or leave watersheds under topographic divides, and thus become inputs and outputs in a water balance equation for a topographically defined watershed.  In other words, be able to give some examples of when groundwater flow directions do not match the slopes of surface topography.

Understand how to think about water balances with rates in different time domains, i.e. daily mean water balance or annual mean water balance.

Understand the influences of hydrologic storage on the basic characteristics of hydrographs.

Understand when or where it is appropriate to apply simplifications to the complete water balance in analysis of a given control volume of the earth surface.

Understand how the runoff ratio can be used to interpret basic partitioning of precipitation into runoff and evapotranspiration.  Understand how comparison of discharge, precipitation, and runoff ratio can be used to compare fundamental watershed inputs and partitioning of those inputs.

Understand how the energy and water balances are integrated through state changes.  Understand how this drives the distribution of water on the planet.

Understand the partitioning of shortwave and longwave radiant energy in the atmosphere and earth surface.  Understand how this is tied to heat (thermal energy) in the atmosphere and earth surface.  Understand the dynamics of longwave radiation, its absorption, and its re-emission that generate the greenhouse effect.

Useful equations:

Areal yield (Y) of a watershed as an area normalized flow.

Y=QAw

where Aw is the area of the watershed generating Q at the outlet.

The generic conservation equation is based on the principle that energy and matter cannot be created or destroyed.

∆S=I-O

where ∆S is the change in the total stored energy or matter within a given control volume during a given period of time, I is the sum of all inputs of energy or matter to that control volume during the same period of time, and O is the sum of all outputs of energy or matter from that control volume during the same period of time.

Full conservation equation for water balance by volume, for any control volume defined on the earth surface.

∆S=P+QIN+GIN-QOUT-EET-GOUT

where ∆S is the change in volume of water stored in the control volume, P is the input of precipitation (from the atmosphere), Q is the input or output of surface water (generally streams or rivers), G is the input or output of groundwater, and EET is the output of evapotranspiration (to the atmosphere).

Full conservation equation for water balance by time-averaged volumetric rates.

∆S∆t=P+QIN+GIN-QOUT-EET-GOUT

where ∆t is the time scale for the average (e.g. 1 day for daily average or 1 year for annual average), and the line above the variable indicates a time-averaged rate (e.g. daily average or annual average).  Note that the dimensions for volumetric rates are frequently the result of normalization to watershed area in water balance analyses (i.e. all terms have dimensions of [L T-1] rather than [L3 T-1]).

Alternative nomenclature for full conservation equation for water balance by time-averaged volumetric rates, as depicted in the Dingman text.

∆S∆t=μP+μQIN+μGIN-μQOUT-μEET-μGOUT

where μ depicts the time-averaged rate rather than the line above the variable in the previous equation.

Simplified water balance for partitioning precipitation into runoff and evapotranspiration.

μP=μQOUT+μEET

Note that the assumptions behind the simplification should be checked before using this simplified water balance.

Definition of the runoff ratio (RRO) as the fraction of precipitation that is partitioned to runoff from the watershed (i.e. discharge at the outlet).

RRO=μQOUTμP

Stefan-Boltzmann law, showing that the total radiant energy emitted by matter (J) due to its thermal properties is proportional to temperature (T, in Kelvins) to the 4th power.

J=ϵσT4

where σ is the Stefan-Boltzmann constant (same for all matter assuming units of T is Kelvins), and ϵ is the efficiency of emission (a.k.a. “emissivity”) that varies with the characteristics of matter (particularly in gasses).  The highest efficiency emitter is a “black body”, which has ϵ=1.