FRANCIS XAVIER ENGINEERING COLLEGE, TIRUNELVELI

(An Autonomous Institution)

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

ACADEMIC YEAR: 2024-25/EVEN			BATCH: 2024-28		SEM: 02

Course Code / Name: 24EE2501 / Fundamental of Electrical and Electronics Engineering

QUESTION BANK

Unit II - ELECTRICAL MACHINES

DC Generator- DC Motor- Single Phase Transformer - single phase induction Motor: Construction, Principle of Operation, EMF Equation and Applications.

Course Outcome – CO2: Discuss the basic operation of electric machines and transformers

Important topics

- Topic 1 – T1- DC Generator: Construction, Principle of Operation, EMF Equation and Applications

- Topic 2 – T2- DC Motor: Construction, Principle of Operation, EMF Equation and Applications

- Topic 3 – T3- Single Phase Transformer: Construction, Principle of Operation, EMF Equation and Applications

- Topic 4 – T4- single phase induction Motor: Construction, Principle of Operation, EMF Equation and Applications.

PART – A

| Q.No | Question | Max. Marks | Topic | CO | BL | KC | PI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | A 4 pole lap wound DC shunt generator rotates at the speed of 1500 rpm, has a flux of 0.4 mWb and the total number of conductors are 1000. Find the value of EMF generated? | 2 | T1 | 2 | K2 | F | 2.2.3 |
| 2. | Describe the role of this component in a DC generator. | 2 | T1 | 2 | K3 | C | 1.3.1 |
| 3. | If speed is decreased in a DC motor, What happens to the back emf and armature current? | 2 | T2 | 2 | K3 | C | 3.1.6 |
| 4. | Discuss the transformation ratio in a transformer | 2 | T3 | 2 | K3 | F | 1.3.2 |
| 5. | List out the various application of induction motor | 2 | T4 | 2 | K2 | C | 1.3.1 |
| 6. | Discuss the slip of an induction motor | 2 | T4 | 2 | K2 | P | 2.2.3 |

PART – B

| Q.No |  | Question | Max. Marks | Topic | CO | BL | KC | PI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | i | Calculate the emf generated by a 6 pole DC generator having 480 conductors and driven at a speed of 1200 rpm. The flux per pole is 0.012wb. Assume the generator to be a) lap wound, b) wave wound. | 6 | T1 | 2 | K4 | P | 3.1.6 |
|  | ii | Derive expression for the emf induced in the DC generator | 7 | T1 | 2 | K4 | P | 3.1.6 |
| 2. |  | A DC motor connected to a 460 V supply has an armature resistance of 0.15 ohm calculate a) value of back emf when the armature current is 120A. b) the value of armature current when the back m is 447 V. | 13 | T2 | 2 | K3 | C | 1.3.1 |
| 3. |  | Describe the working principle and Derive the mathematical expression for back electromotive force (back emf) in a DC motor | 13 | T2 | 2 | K3 | C | 3.1.6 |
| 4. |  | Illustrating the principle of operation and EMF equation for the above component to convert AC to AC without change the Frequency. | 13 | T3 | 2 | K4 | F | 3.2.2 |
| 5. |  | Describe the following types of induction motor with neat diagram, advantages and disadvantages and its applications. 1)Split phase motor 2)Shaded pole motor | 13 | T4 | 2 | K3 | P | 3.1.6 |

PART – C

| Q.No |  | Question | Max. Marks | Topic | CO | BL | KC | PI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Name the bac emf which opposes the supply voltage in DC Motor and Illustrate how the Torque equation generated in DC motor. | 15 | T2 | 2 | K3 | C | 3.2.2 |

| Topic | Part A |  | Part B |  | Part C |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | No. of Questions | Total Marks | No. of Questions | Total Marks | No. of Questions | Total Marks |
| Topic 1 - Performance of short line, medium line and long line | 1 | 2 | 2 | 26 |  |  |
| Topic 2 – equivalent circuits, phasor diagram, attenuation constant, phase constant, surge impedance | 2 | 4 | 1 | 13 |  |  |
| Topic 3 – transmission efficiency and voltage regulation, real and reactive power flow in lines. | 1 | 2 | 1 | 13 | 1 | 15 |
| Topic 4 – Power Circle diagrams – Phenomenon of Corona– Ferranti Effect. | 2 | 4 | 1 | 13 |  |  |
| TOTAL | 6 | 12 | 5 | 65 | 1 | 15 |